# Blockchain-Based IoT Edge Computing System

## Engineering Installation & Operation Manual

## 1. Purpose of This Manual

This document is a technical installation and usage manual, written the way industrial software or field-deployed systems are documented.

It explains:

- Why each technology was selected

- How the system is installed and deployed

- What actually happens under the hood when the system runs

- How data flows from device → edge → blockchain → dashboard

- How to operate, validate, and troubleshoot the system

This is a workthrough manual for reviewers.

## 2. System Objective

The project demonstrates a secure, decentralized Industrial IoT edge architecture where:

- IoT devices publish data to an edge gateway

- The gateway validates and rate-controls data

- Critical events are immutably logged on a blockchain

- A web dashboard visualizes device state and blockchain confirmations

The core engineering problem addressed:

How do you ensure integrity, traceability, and trust in edge-generated IoT data without relying on a centralized server?

## 3. High-Level Architecture

Logical Stack

- IoT Edge Devices – ESP32 / Arduino-class MCU

- Message Transport Layer – MQTT Broker (Mosquitto)

- Edge Gateway – Node.js validation & blockchain interface

- Blockchain Layer – Ethereum-compatible local network (Hardhat)

- Smart Contract – Immutable event logging & verification

- Frontend Dashboard – React-based visualization

Each layer is intentionally isolated to reflect real industrial deployment boundaries.

## 4. Technology Selection & Engineering Rationale

### 4.1 Microcontroller (ESP32 / Arduino)

Reasoning:

- Sufficient processing for cryptographic hashes

- Native WiFi support

- Widely used in industrial prototyping

- Deterministic firmware behavior

Design Decision: The MCU does not interact directly with the blockchain.

Blockchain interaction is intentionally offloaded to the edge gateway to avoid latency, gas cost exposure, and firmware complexity.

### 4.2 MQTT (Mosquitto Broker)

Why MQTT:

- Lightweight publish/subscribe model

- Decouples device and backend

- Industry-standard for IIoT

Why Mosquitto:

- Open-source

- Minimal overhead

- Easy local deployment

Operational Logic:

- Devices publish sensor or event payloads

- Gateway subscribes to validated topics

- No direct device-to-dashboard coupling

### 4.3 Edge Gateway (Node.js)

Role: This is the trust enforcement layer.

Responsibilities:

- MQTT subscription

- Payload validation

- Timestamp normalization

- Nonce-based replay protection

- Rate limiting

- Blockchain transaction orchestration

Why Node.js:

- Strong async I/O for MQTT

- Mature blockchain libraries (ethers.js / web3.js)

- Clean separation of logic layers

### 4.4 Blockchain Network (Hardhat – Local Ethereum)

Purpose:

- Provide immutable logging

- Demonstrate smart contract interaction

- Avoid dependency on public networks

Why Local Blockchain:

- Zero gas cost

- Deterministic testing

- Full control over accounts and mining

The focus is system trust logic, not public chain deployment.

### 4.5 Smart Contract

Function:

- Accept validated event data

- Emit on-chain events

- Store hashes / metadata immutably

Key Design Choice: The contract does not store raw sensor data.

Instead, it stores:

- Hashes

- Device IDs

- Timestamps

This mirrors real-world blockchain constraints.

### 4.6 Frontend Dashboard (React)

Purpose:

- Human-readable visualization

- Event confirmation display

- Blockchain transaction tracking

Design Principle: Frontend is read-only with respect to blockchain state.

All authority remains at the edge gateway.

## 5. Installation Prerequisites

### Software

- Node.js (LTS)

- npm

- Hardhat

- Mosquitto MQTT Broker

- Git

### Hardware

- ESP32 or Arduino-compatible board

- WiFi network

## 6. Repository Structure

/BLOCKCHAIN_BASED_IOT_EDGE
│
├── docs,demo_videos          
├── Trust_Layer/                   # Deployment scripts
            |----contracts/        # Solidity smart contracts
            |----sripts/           # Hardhat based Node deployment
            |----hardhat.config.js
├── iot-gateway/                   # Node.js MQTT + blockchain logic
            |----.env              # Environment configuration
            |----contract-abi.json # ABI generated in trust layer artifacts
            |----gateway.js        # gateway deployment for IOT and blockchain comm
├── MCU_nodes/                     # MCU code
├── web-dashboard/src              # React dashboard
            |----components         #app components
            |----utils/

|-------web3Config.js #configuration for dashboard deployment

## 7. System Installation & Run Procedure (End-to-End)

This section documents the complete operational sequence from Solidity source code to a live React dashboard. Each step explains what is executed, what artifacts are produced, and why the step exists in the system.

### Step 1: Clone Repository

git clone <your-repo-url>
cd BLOCKCHAIN_BASED_IOT_EDGE

This retrieves all system layers (contracts, gateway, MCU firmware, and frontend) into a single, version-controlled workspace.

### Step 2: Install Node Dependencies (Root & Submodules)

npm install

This installs Hardhat, ethers.js, and supporting tooling required for Solidity compilation and deployment.

Why this matters: Hardhat acts as the local blockchain runtime and compiler toolchain, replacing external services like Ganache.

### Step 3: Compile Solidity Smart Contracts

npx hardhat compile

What happens internally: - Solidity source files in contracts/ are parsed - Bytecode is generated - ABI (Application Binary Interface) is produced - Artifacts are written to artifacts/ directory

Why ABI exists: The ABI is the contract’s public interface. It is required by: - Edge gateway (Node.js) - React dashboard

Without the ABI, off-chain code cannot legally encode or decode contract calls.

### Step 4: Start Local Blockchain (Hardhat Node)

npx hardhat node

This launches a local Ethereum-compatible blockchain that provides: - Deterministic block mining - Pre-funded test accounts - Stable RPC endpoint (http://127.0.0.1:8545)

Engineering rationale: Hardhat Node replaces Ganache because it: - Is natively aligned with Hardhat artifacts - Avoids RPC instability issues - Provides deterministic account management

The blockchain exists in-memory, but persists while the node is running.

### Step 5: Deploy Smart Contract

In a new terminal:

npx hardhat run scripts/deploy.js --network localhost

What happens: - Deployment account signs transaction - Contract bytecode is sent to Hardhat node - Contract is mined into a block - A unique contract address is generated

Example output:

IoTDataLogger deployed to: <DEPLOYED_CONTRACT_ADDRESS>
Contract owner: <DEPLOYER_ADDRESS>

This address is the only authoritative reference to the deployed contract.

### Step 6: Extract and Distribute ABI

Copy ABI file:

artifacts/contracts/IoTDataLogger.sol/IoTDataLogger.json

Copy only the abi section into: - iot-gateway/contract-abi.json - web-dashboard/src/utils/contractABI.json

Why this step is explicit: This enforces interface consistency across backend and frontend. ABI mismatch errors occur when this step is skipped or stale files are used.

### Step 7: Configure Gateway Environment

Edit iot-gateway/.env:

RPC_URL=http://127.0.0.1:8545
PRIVATE_KEY=<DEPLOYER_PRIVATE_KEY>
CONTRACT_ADDRESS=<DEPLOYED_CONTRACT_ADDRESS>
MQTT_BROKER=localhost
MQTT_PORT=1883

Design reasoning: - Gateway owns the signing key - Devices and dashboard never access private keys - All blockchain authority is centralized at the edge

### Step 8: Configure Dashboard Web3 Binding

Edit web-dashboard/src/utils/web3Config.js:

- Set RPC provider to Hardhat node

- Set deployed contract address

- Import correct ABI

Purpose: The dashboard is a read-only blockchain client, used only to visualize on-chain state and events.

### Step 9: Start MQTT Broker with custom made mosquito.conf file to allow anonymous users to communication through firewall which the default routine does not

mosquito -c mosquitto.conf -v

This enables device-to-gateway communication.

### Step 10: Start Edge Gateway

cd iot-gateway
npm install
node gateway.js

Runtime behavior: - Subscribes to MQTT topics - Validates payload structure - Applies rate limiting and nonce checks - Submits blockchain transactions

### Step 11: Flash MCU Firmware

MCU publishes structured JSON payloads via MQTT.

Key principle: Devices are intentionally blockchain-unaware.

### Step 12: Start React Dashboard

cd web-dashboard
npm install
npm start

Dashboard connects to: - Hardhat RPC - Deployed contract

Live blockchain events are visualized in real time.

## 8. End-to-End Data Flow (What Really Happens)

- Device publishes MQTT message

- Gateway receives payload

- Payload validated

- Hash computed

- Blockchain transaction sent

- Smart contract emits event

- Frontend detects event

- User sees confirmation

No step bypasses validation.

## 9. Failure Modes & Engineering Safeguards

| Failure | Mitigation |
| --- | --- |
| MQTT flood | Rate limiting |
| Replay attack | Nonce tracking |
| Blockchain delay | Async queue |
| Device spoofing | ID whitelisting |

## 10. Design Limitations (Explicitly Acknowledged)

- Not optimized for high throughput

- Local blockchain only

- No hardware secure element

These are conscious engineering tradeoffs, not omissions.

## 11. Engineering Value Summary

This project demonstrates:

- Edge trust enforcement

- Blockchain integration without hype

- Practical IIoT architecture

- Separation of concerns

- Security-aware design

It is engineering-first, not trend-driven.

## 12. How This Would Scale in Industry

- Replace Hardhat with private Ethereum / Hyperledger

- Add TLS + cert-based MQTT

- Introduce hardware root-of-trust

- Deploy gateway in containerized edge node

End of Manual